Louisiana Small Engines CDE TestBank

Test ID: 74

Name: ___________________________     School: __________________________

Directions:   Select the Best Answer.

1. When a small gas engine refuses to start, the first check should be:

(A). Spark plug gap setting

(B). Crankcase oil level

(C). Piston clearance

(D). Level of the fuel in the tank

2. Air cooled engines can be recognized by:

(A). The cooling fins on the cylinder

(B). Large radiators

(C). Smoking exhaust

(D). Slower operating speeds

3. You have removed the spark plug and it is wet with gasoline, the engine won't start and no other checks have been made.  It can be assumed that:

(A). Flooding is the problem

(B). A weak spark is the problem

(C). Either flooding or a weak spark could be the problem

(D). Neither flooding or a weak spark could be the problem

4. If the engine starts readily but runs only a few minutes, the most likely cause of this problem is:

(A). Poor piston and cylinder lubrication

(B). The fuel flow from the tank to the carburetor may be slow or irregular

(C). Spark plug is bridged with carbon

(D). Weak or discharged condenser

5. Most small gas engines have a __________ type lubrication.

(A). Wick

(B). Splash

(C). Full flow

(D). Positive displacement oil pump

6. Fuels stored for a long period of time in the carburetor may contain:

(A). Excess gum and varnish

(B). Excess hydrocarbons

(C). Excess lead

(D). Calcium chloride

7. If the lawnmower engine vibrates excessively while running, the cause may be:

(A). Bent crankshaft

(B). Blade out-of-balance

(C). Bent blade

(D). Any of the above

8. You check the ignition system with the ignition tester and there is no spark.  The most likely cause of this problem is:

(A). Sheared flywheel key

(B). Grounded / bad coil

(C). Shorted stop switch

(D). Any of the above

9. Which one of the following is not a type of manual starter on a small gas engine?

(A). Rope-wind

(B). Clutch and ratchet

(C). Rope-rewind

(D). Wind-up

10. Weak compression can be a result of:

(A). Worn spark plug

(B). Worn rings

(C). Worn breaker points

(D). Worn connecting rod

11. A spark plug with a soft-tan-powdery deposit is an indication of:

(A). Burned electrodes

(B). Lead fouling

(C). Oil fouling

(D). Carbon fouling

12. A spark plug with a wet-oily deposit is an indication of:

(A). Burned electrodes

(B). Lead fouling

(C). Oil fouling

(D). Carbon fouling

13. When measuring tappet clearance, one should use:

(A). A flat feeler gauge

(B). A round feeler gauge

(C). Plasti-gauge

(D). An inside micrometer

14. For preventive maintenance, the gasoline fuel system should be emptied and cleaned and refilled with fuel every                           .

(A). 25 hours of engine operation

(B). 50 hours of engine operation

(C). 75 hours of engine operation

(D). 100 hours of engine operation

15. During normal operating conditions, the small gas engine oil should be changed every:

(A). 10 hours

(B). 25 hours

(C). 50 hours

(D). 100 hours

16. The air pressure against the air vane tends to _____________ the throttle.

(A). Close

(B). Open

(C). Neither A or B

(D). Either A or B

17. The initial adjustment on the Briggs & Stratton flo-jet needle valve is made by turning it __________ turn(s) off the seat.

(A). One half

(B). One

(C). One and one half

(D). Two

18. Oil foam air cleaners should be cleaned with:

(A). Diesel fuel

(B). Gasoline

(C). Detergent and water

(D). Kerosene

19. Valves on a 4 cycle small gas engine should be replaced when the original margin thickness has worn more than:

(A). One-fourth the thickness of the margin

(B). Three-fourths the thickness of the margin

(C). One-third the thickness of the margin

(D). One-half the thickness of the margin

20. Increasing governor spring tension:

(A). Reduces the load on the engine

(B). Increases the speed of the engine

(C). Increases the load on the engine

(D). Reduces both engine load and speed

21. The engine that you are working on tends to kick back when you try to start it. The  problem could be:

(A). The valve clearances are too close

(B). The valve clearances are too wide

(C). The head bolts have too much torque

(D). The sparkplug torque is incorrect

22. If a seat is too wide:

(A). Carbon will build up

(B). The valve will run too cool

(C). The stem will bend

(D). The head will warp

23. When reinstalling the cap to the connecting rod, it is most important to align the:

(A). Match marks

(B). Timing marks

(C). Valve clearances

(D). Window

24. A lot of 4 cycle small gas engines have an oil ___________  attached to the end of the connecting rod.

(A). Shovel

(B). Injector

(C). Dipper

(D). Heater

25. The part which connects the piston and the connecting rod is known as the:

(A). Push pin

(B). Connecting pin

(C). Piston pin

(D). Dowel pin

26. Which of the following is not a type of valve spring retainer?

(A). Pin

(B). Ball

(C). Slotted disc

(D). Split collar

27. Fuel volatility refers to:

(A). Evaporation heat

(B). Octane rating

(C). Vapor pressure

(D). Viscosity

28. Maintenance of the small gas engine includes:

(A). An oil change

(B). Air filter cleaning or replacement

(C). Cleaning and gapping the spark plug

(D). All of the above

29. Briggs & Stratton engine 126332 uses a ___________ governor to control engine speed.

(A). Pneumatic

(B). Electric

(C). Mechanical

(D). Hydraulic

30. Cleaning the air cooling systems is a routine maintenance item.

(A). True

(B). False

31. When storing a small gas engine, it is recommended to set the piston at top dead center of the exhaust stroke.

(A). True

(B). False

32. When storing an engine over a season, drain the crankcase oil and refill with new oil of the recommended type and viscosity.

(A). True

(B). False

33. The mating surface of the valve face is:

(A). The valve seat

(B). The valve stem

(C). The valve head

(D). The valve margin

34. The middle ring on the 4 cycle gas engine piston is called a/an:

(A). O-ring

(B). Compression and/or scrapper

(C). Oil ring

(D). Teflon

35. During the compression stroke on the 4 cycle engine:

(A). The exhaust valve closes and the piston moves upward

(B). The exhaust valve opens and the piston moves upward

(C). The intake valve opens and the piston moves upward

(D). The intake valve closes and the piston moves upward

36. The exhaust valve opens and the upward movement of the piston forces burned gasses out of the cylinder on the:

(A). Intake stroke

(B). Compression stroke

(C). Power stroke

(D). Exhaust stroke

37. The perforated ring on a small gas engine piston is the:

(A). Compression ring

(B). Oil ring

(C). Scrapper ring

(D). Second compression ring

38. A clogged crankcase breather will cause:

(A). The engine to overheat

(B). The governor to malfunction

(C). Leaking oil seals and ruptured gaskets

(D). Negative crankcase pressure

39. The purpose of the cylinder wall crosshatch pattern is to:

(A). Control lubrication of the cylinder and aid in seating new rings

(B). Reduce combustion temperature

(C). Make the piston and rings slide easier

(D). Cool crankcase lubricant

40. Valve tappet clearance is needed to:

(A). Allow valves to expand

(B). Allow valves to stay open longer

(C). Reduce wear on the stem

(D). Reduce wear on the valve face

41. The greatest amount of cylinder wear is at the:

(A). Top one-eighth of the cylinder wall

(B). Middle one-third of the cylinder wall

(C). The bottom one-third of the cylinder wall

(D). The top one-third of the cylinder wall

42. The 4 cycle gas engine intake valve is cooled by:

(A). Air from the flywheel fins

(B). Oil from the crankcase

(C). Air circulating around the outside of the case of the engine

(D). Incoming air fuel mixture

43. Which of the following is not a part of the valve?

(A). Head

(B). Margin

(C). Seat

(D). Face

44. End clearance on rings is necessary so that:

(A). Exhaust gases may escape the combustion chamber after the compression stroke

(B). The piston rings will have room to expand when it becomes hot

(C). The ring can exert pressure on the cylinder wall

(D). Both B & C

45. After cylinders are bored or honed they should be cleaned with

(A). Gasoline

(B). Kerosene

(C). Soapy water

(D). Either B or C

46. The top ring on a piston in a small gas engine is called a/an:

(A). Exhaust ring

(B). Oil ring

(C). Compression ring

(D). Scrapper ring

47. The average operating temperature of the intake valve is approximately __________ degrees Fahrenheit.

(A). 12,000

(B). 1,200

(C). 120

(D). 12

48. Valve sticking can be caused by:

(A). Lead in the fuel

(B). Gum in the fuel

(C). Varnish from the fuel

(D). All of the above

49. The terms flo jet, pulsa jet, and vacu jet are used to describe the three types of:

(A). Injectors

(B). Venturi tubes

(C). Carburetors

(D). Needle valves

50. The angle the valve face and seat on a Briggs & Stratton engine are ground is generally:

(A). 45 and 30 degrees

(B). 45 and 50 degrees

(C). 15 and 30 degrees

(D). 60 and 45 degrees

51. The compression ratio tells us about the engine's:

(A). Horsepower

(B). Torque

(C). Efficiency

(D). All of the above

52. The cam lobes:

(A). Are timed with the camshaft gear so the valves are lifted at the correct time

(B). Contour determines the speed of the valve opening

(C). Contour determines the length of time the valves are open

(D). All of the above

53. Which of these is not a reason for valve failure?

(A). Burned

(B). Dished

(C). Carboned

(D). Necked

54. The first one or two numbers of the model number system indicates:

(A). Horsepower

(B). Oil capacity

(C). Cubic Inch Displacement

(D). Position of the crankshaft and carburetor

55. If there is a difference in the size of the valve head, the intake valve is __________ than the exhaust valve.

(A). The same size

(B). Smaller

(C). Larger

56. Valves have to seal well enough to stand pressures up to __________ pounds per square inch.

(A). 250 PSI

(B). 500 PSI

(C). 750 PSI

(D). 1000 PSI

57. The function of the crankcase breather is to remove __________ from the crankcase.

(A). Oil

(B). Air

(C). Dirt

(D). Gum

58. For the valves to open at the proper time the __________ marks must be aligned.

(A). Spring

(B). Hobbing

(C). Timing

(D). Plug

59. The compression release is designed to:

(A). Reduce Vibration

(B). Prevent Backfiring

(C). Shut off the engine

(D). Allow Easier Starting

60. Valves are held in the closed position by valve guides.

(A). True

(B). False

61. Weak compression may be caused by a leaky head gasket.

(A). True

(B). False

62. Weak compression could not be caused by worn rings.

(A). True

(B). False

63. Cylinders can be resized with a hone.

(A). True

(B). False

64. Which part of the magneto ignition system converts low voltage into high voltages?

(A). Armature Laminations

(B). Permanent Magnets

(C). Condenser

(D). Coil

65. The most likely problem with an engine that runs rich and blows black smoke is:

(A). Low compression

(B). Restricted air intake

(C). Tight valves

(D). Weak ignition

66. The purpose of proper ignition timing is to

(A). Open and close the intake valve at the precise time

(B). Produce spark at the precise moment

(C). To prevent air pollution

(D). To reduce oil consumption

67. The secondary winding of the magneto coil has approximately __________ times as many turns of wire as the primary coil circuit.

(A). 30

(B). 60

(C). 90

(D). 120

68. If you notice fuel dripping from the carburetor. The problem could be:

(A). The fuel line is clogged.

(B). The float is stuck in the closed position

(C). The float is stuck in the open position

(D). The fuel valve is in the on position

69. The relatively low voltage produced in the primary coil windings is increased by the:

(A). Lead wire going to the spark plug

(B). Rapid spinning of the flywheel

(C). Induced voltage in the secondary circuit

(D). Action of the condenser

70. The magneto primary coil consists of:

(A). Few windings of heavier wire

(B). Few windings of very fine wire

(C). Many windings of very fine wire

(D). Many windings of heavier wire

71. Flo jet carburetors must have the float adjusted or set __________ to operate properly.

(A). Level

(B). Vertical

(C). Horizontal

(D). Both A & C

72. A loss of spark when the engine reaches operating temperature generally indicates a:

(A). defective coil

(B). defective condenser

(C). ruptured diaphragm

(D). faulty air cleaner

73. Which of the following spark plug tip insulator descriptions best indicates a "hot" spark plug?

(A). Long

(B). Thin

(C). Thick

(D). Short

74. Which of the following is a part of the ignition coil?

(A). Air Gap

(B). Condenser

(C). Primary winding

(D). Breaker points

75. What spark plug appearance clearly indicates the possibility of too "lean" a carburetor fuel mixture for the 2 cycle engine?

(A). Dry or damp black soot on the insulator or shell

(B). Burned and badly eroded electrode

(C). Light tan or brown insulator

(D). Insulator has gray or white blistered appearance and a burned electrode

76. You remove the float bowl from the engine that you are repairing and no fuel is present:

(A). The fuel line could be pinched

(B). The fuel valve is in the on position

(C). The fuel cap is too tight

(D). The fuel tank is over filled with fuel

77. The ignition coil of a gas engine adjusts the secondary circuit current for the spark plugs by increasing the:

(A). Voltage

(B). Resistance

(C). Amperage

(D). Frequency

78. Which of the following describes the secondary winding of a magneto coil?

(A). Many turns of fine wire

(B). A few turns of fine wire

(C). A few turns of heavy wire

(D). Many turns of heavy wire

79. The main function of the coil is to:

(A). Reduce the voltage

(B). Reduce the current

(C). Increase the voltage

(D). Increase the current

80. When checking the Briggs & Stratton magneto ignition system for spark, it must jump a gap of:

(A). 0.750"

(B). 0.250"

(C). 0.166"

(D). 0.500"

81. The ring gap reject size for a cast iron engine’s oil ring is:

(A). .035

(B). .040

(C). .030

(D). .028

82. A sheared or partially sheared flywheel key would cause:

(A). Ignition timing to be off

(B). Valve timing to be off

(C). PTO shaft slippage

(D). An increase in voltage at the spark plug

83. During the intake stroke, the exhaust valve is:

(A). Deformed

(B). Hooked

(C). Opened

(D). Closed

84. The part which most frequently breaks on the recoil starter unit is the:

(A). Rope

(B). Spring

(C). Guide Lug

(D). Hub

85. Vapor lock can occur when:

(A). The fuel gets hot

(B). Water gets in the fuel system

(C). The radiator overheats

(D). Oil gets into the combustion chamber

86. At low speeds the ignition spark should be supplied closer to TDC.

(A). True

(B). False

87. A four stroke cycle single cylinder engine has one power stroke every four revolutions of the camshaft.

(A). True

(B). False

88. Solid state ignition systems eliminate the conventional breaker points.

(A). True

(B). False

89. The crankshaft in a 4 cycle engine turns at ____________ the speed of the camshaft.

(A). Twice

(B). Four times

(C). One-half

(D). The same speed as

90. The 4 cycle single cylinder engine has a power stroke ____________ revolution of the crankshaft.

(A). Every

(B). Every other

(C). Every third

(D). Twice per

91. The two materials most widely used in manufacturing small gas engine blocks are:

(A). Magnesium and cast iron

(B). Copper and Aluminum

(C). Aluminum and cast iron

(D). Tin and zinc

92. When an engine is operating at 3000 RPM each valve opens and closes in approximately:

(A). 1 second

(B). 1/10 of a second

(C). 1/25 of a second

(D). 1/50 of a second

93. An excess of carbon accumulates in the combustion chamber and it causes an increase in the:

(A). Clearance volume

(B). Compression ratio

(C). Stroke

(D). Piston Displacement

94. In the 4 cycle small gas engine, how many complete revolutions must the crankshaft make to complete one cycle?

(A). One

(B). Two

(C). Three

(D). Four

95. The ring end gap measures 0.045" and the reject size is 0.035".  The rings:

(A). Are still satisfactory

(B). Must be replaced

(C). Should be ground down

(D). Telescoping gauges

96. Which of the following indicates the proper piston stroke sequence for a 4 cycle engine?

(A). Intake, Power, Exhaust, Compression

(B). Intake, Compression, Exhaust, Power

(C). Intake, Compression, Power, Exhaust

(D). Intake, Power, Compression, Exhaust

97. Piston displacement refers specifically to the:

(A). Diameter of the cylinder

(B). Length of the piston stroke

(C). Area of the top of the piston

(D). Space in the cylinder passed through by the piston

98. Engine compression is measured in:

(A). Lbs/square inch

(B). KG/SQ cm

(C). Either of the above

(D). None of the above

99. Viscosity index indicates:

(A). A sequence number for fuel

(B). A micrometer reference point

(C). A characteristic of oil

(D). The pressure developed by the fuel pump

100. Which of the following is not one of the strokes of the 4 cycle engine?

(A). Compression

(B). Vacuum

(C). Exhaust

(D). Intake

101. When a piston has reached the top of its stroke, it is known as:

(A). IRS

(B). ABC

(C). BDE

(D). TDC

102. The connecting rod serves as the link between the:

(A). Crankshaft

(B). Crankshaft and cam journal

(C). Piston and crankshaft

(D). Camshaft and piston

103. When air passes through the carburetor venture:

(A). A.Velocity and pressure increases

(B). B.Velocity increases and pressure decreases

(C). C.Velocity and pressure decreases

(D). D.Velocity increases and pressure remains constant

104. Atmospheric pressure can vary slightly because of:

(A). Altitude

(B). Temperature

(C). Both A and B

(D). None of the above

105. A general rule of thumb to follow is to allow a clearance between the cylinder and piston of __________ per cylinder diameter.

(A). 0.005 inch

(B). 0.001 inch

(C). 0.0001 inch

(D). 0.01 inch

106. Stroke is the total distance moved by the piston:

(A). In one-half revolution of the crankshaft

(B). In one revolution of the crankshaft

(C). In two revolutions of the crankshaft

(D). In three revolutions of the crankshaft

107. During the intake stroke, the piston is moving:

(A). Towards top dead center

(B). Sideways

(C). Away from top dead center

(D). None of the above

108. During the intake stroke the intake valve is:

(A). Deformed

(B). Closed

(C). Opened

(D). None of the above

109. During the compression stroke the intake valve is:

(A). Opened

(B). Milled off

(C). By passed

(D). None of the above

110. During the compression stroke the exhaust valve is:

(A). Opened

(B). Closed

(C). By passed

(D). Lapped

111. There are __________ strokes in a cycle with the 4 cycle engine.

(A). 2

(B). 4

(C). 6

(D). 8

112. The primary purpose of a crankshaft is to change ____________ motion into rotary motion.

(A). Reciprocating

(B). Rotary

(C). Crosswise

(D). Off set

113. The ____________ turns the crankshaft.

(A). Cylinder head

(B). Piston and connecting rod

(C). Cam gear and cam shaft

(D). Valve lifters and twisters

114. Camshafts are usually made of:

(A). Plastic

(B). Cast Iron

(C). Brass

(D). A or B

115. The single cylinder engine has __________ lobes on the camshaft.

(A). 1

(B). 2

(C). 3

(D). 4

116. The flywheel must have a __________ metal key to prevent crankshaft damage.

(A). Cast iron

(B). Alloy

(C). Soft

(D). Hard

117. The compression ratio of an engine is:

(A). The proportion or ratio of the piston stroke to the cylinder bore

(B). The ratio of total volume in the cylinder to the head clearance volume

(C). The ratio of cylinder volume displaced by the piston at BDC and TDC

(D). Ratio of air to fuel compressed in the cylinder on the compression stroke

118. The main purpose of the camshaft lobe and tappet is to:

(A). Open the valves

(B). Propel the crankshaft

(C). Move the piston up and down

(D). Pump oil

119. Increasing the valve tappet clearance on a small engine can be done by:

(A). Grinding the valve seat

(B). Grinding the end of the valve stem

(C). Grinding the valve face

(D). Changing the valve guides

120. Engine horsepower will:

(A). Decrease 3 percent for each 1000 feet increase above 1000 feet of elevation

(B). Decrease 3 percent for each 1 inch drop in barometric pressure

(C). Decrease approximately 1 percent for each 10 degrees rise in temperature

(D). All of the above

121. A small gas engine is approximately __________ efficient.

(A). 25 %

(B). 50 %

(C). 75 %

(D). 100 %

122. The purpose of the carburetor float is to regulate:

(A). Air flow

(B). Engine speed

(C). Engine heat

(D). Fuel level

123. Must manufacturers recommend servicing the air filter every __________ hours of operations under normal conditions.

(A). 10

(B). 15

(C). 25

(D). 50

124. In general the worst enemy of the small gas engine is:

(A). Paint

(B). Hours of operation

(C). Heat

(D). Dust

125. The basic purpose of a carburetor is to:

(A). Clean the air entering the engine

(B). Regulate the mixture of air and fuel

(C). Regulate the amount of fuel entering the engine

(D). Equalize the amount of fuel entering the engine

126. The choke reduces the amount of __________ flow through the carburetor.

(A). Fuel

(B). Air

(C). Dirt

(D). Coolant

127. To compensate for changing speeds or loads the governor on an engine adjusts the:

(A). Throttle opening

(B). Air-fuel ratio

(C). Breaker point gap

(D). Ignition timing

128. Too little crankcase oil in the 4 cycle engine will cause:

(A). A seized engine

(B). Fouling spark plugs

(C). Fast idling

(D). Longer engine life

129. Two types of small gas engine governors are:

(A). Hand operated and limited fuel supply

(B). Limited fuel supply and free-wheeling

(C). Pneumatic and mechanical

(D). Friction and frictionless

130. Fuel flow into the Briggs & Stratton flo-jet carburetor bowl is accomplished by:

(A). Atmospheric pressure

(B). Gravity

(C). Fuel pump

(D). None of the above

131. The Briggs & Stratton pulsa-jet type carburetor adjustments include:

(A). Needle valve and idle speed

(B). Needle valve

(C). Needle valve and idle valve

(D). None of the above

132. Briggs & Stratton carburetors are described as:

(A). Flo-jet or gravity or float type

(B). Vacu-jet or suction feed type

(C). Pulsa-jet or fuel pump type

(D). All of the above

133. Mechanical governors use a spring and ____________ to maintain a constant speed.

(A). Air vane

(B). Counterweights

(C). Counter clockwise

(D). Chain weights

134. The recommended Briggs & Stratton idle speed is:

(A). 500 RPM

(B). 1075 RPM

(C). 1750 RPM

(D). 2150 RPM

135. The idle speed is set by the:

(A). Idle mixture screw

(B). Idle adjusting screw

(C). Needle valve

(D). Float needle

136. For Briggs & Stratton small gas engines using regular gasoline the most efficient compression ratio is:

(A). Ten to one or eleven to one

(B). Three to one or four to one

(C). Five to one or six to one

(D). Twenty to one or twenty-one to one

137. The initial carburetor adjustment is:

(A). Choke

(B). Throttle

(C). High speed mixture

(D). Idle mixture

138. The air fuel mixture is ____________ when it passes through the intake manifold and enters the cylinder combustion chamber.

(A). Atomized

(B). Moisturized

(C). Vaporized

(D). Naturalized

139. The diaphragm type carburetor has a rubber like diaphragm with:

(A). Intake manifold pressure acting on both sides

(B). Atmospheric manifold pressure acting on both sides

(C). Intake manifold pressure on one side and atmospheric pressure on the other

(D). None of the above

140. At the bottom of the intake stroke the intake valve closes.

(A). True

(B). False

141. During the compression stroke both valves are open.

(A). True

(B). False

142. Just before compression ends, the magneto produces a high voltage arc across the spark plug gap igniting the air fuel mixture.

(A). True

(B). False

143. During the power stroke, the rapid burning mixture produces a very high pressure to push the piston down.

(A). True

(B). False

144. Exhaust gases contain ____________, an odorless deadly poison.

(A). Carbon Dioxide

(B). Carbon Trioxide

(C). Carbon Monoxide

(D). None of the above

145. Do not fill the gas tank while the engine is running.

(A). True

(B). False

146. Fresh gasoline minimizes:

(A). Gum deposits

(B). Carbon deposits

(C). Overheating

(D). Oil consumption

147. You should use a 10W-30 or 30 weight oil.

(A). True

(B). False

148. Oil should be changed monthly or every __________ hours of operation.

(A). 100

(B). 10

(C). 150

(D). 50

149. Dust particles from the air cannot harm the oil.

(A). True

(B). False

150. Fresh oil assists in cooling.

(A). True

(B). False

151. Engine cleaning should be performed yearly or every __________ hours.

(A). 6

(B). 12

(C). 100

(D). 200

152. To check compression:

(A). Place your thumb over the spark plug hole and pull the crank; if compression is satisfactory it will force your thumb off the hole

(B). Spin the flywheel in reverse rotation, counterclockwise; the flywheel should rebound sharply

(C). Bring it to a small engines shop

(D). Both A and B

153. You remove the fuel line from the carburetor and fuel drips out slowly. You loosen the fuel tank cap and the fuel starts to flow. What is the most likely problem with this engine?

(A). The fuel valve is in the on position

(B). The fuel line is pinched

(C). The fuel line is clogged

(D). The vent in the fuel tank cap is clogged

154. The spark plug gap should be:

(A). .060"

(B). 0.30"

(C). .030"

(D). 0.60"

155. To adjust the valve lash on an OHV engine, you should:

(A). Loosen the set screw, tighten / loosen nut to adjust lash, check clearance, & tighten the set screw.

(B). Check the lash / clearance, tighten or loosen the nut, recheck the clearance.

(C). Put in a spacer and check the lash.

(D). None of these.

156. The armature gap should be:

(A). .010"

(B). .10"

(C). .001"

(D). .100"

157. The flywheel should be torqued to:

(A). 55 in-lbs

(B). 55 ft-lbs

(C). 100 ft-lbs

(D). 100 in-lbs

158. The head bolts should be torqued to:

(A). 100 cm-lbs

(B). 100 ft-lbs

(C). 210 in-lbs

(D). 140 in-lbs

159. The rod cap should be torqued to:

(A). 140 in-lbs

(B). 140 ft-lbs

(C). 100 in-lbs

(D). 55 ft-lbs

160. Removal of the flywheel is not required to remove magnetron, magnavac armatures except to:

(A). Inspect the flywheel fins and magnets

(B). Inspect the flywheel key and keyways

(C). Inspect the armatures

(D). All of the above

161. In the magnetron ignition, the __________ is controlled by the flywheel key.

(A). Timing

(B). Injection

(C). Piston

(D). Valves

162. A properly serviced air cleaner protects the internal parts of the engine from dust particles in the air.

(A). True

(B). False

163. Clean and re-oil the cleaner element every _____ hours or at _____ month intervals under normal conditions.

(A). 25 / 25

(B). 3 / 25

(C). 25 / 3

(D). 3 / 3

164. You should use petroleum solvents to clean paper cartridge air cleaners.

(A). True

(B). False

165. The diaphragm is suitable for further use if it has developed wear spots or punctures.

(A). True

(B). False

166. All carburetor adjustments should be made with the air cleaner on the engine.

(A). True

(B). False

167. On engines with a needle valve, the needle valve should be adjusted __________ turn(s).

(A). 1 1/4

(B). 2

(C). 1 ½

(D). 1

168. Flooding can occur if the engine is tipped at an angle for a prolonged period of time.

(A). True

(B). False

169. On vacu-jet carburetors there is a ____________ in the fuel pickup tube.

(A). Check ball

(B). Stop ball

(C). Flow ball

(D). None of the above

170. The two types of throttles are:

(A). Cast and metal

(B). Cast and stamped

(C). Stamped and round

(D). Round and flat

171. Carburetor adjustments should be made with the tank half full.

(A). True

(B). False

172. You should check the needle valves for burrs, grooves, or bent needle tips.

(A). True

(B). False

173. The three types of remote control are:

(A). Governor Control, Throttle Control, Spark-A-Matic

(B). Throttle Control, Governor Spring, Spark-A-Matic

(C). Spark-A-Matic, Governor Spring, Throttle Spring

(D). Governor Control, Throttle Control, Choke-A-Matic

174. The purpose of the governor is to:

(A). Maintain, within certain limits, a desired engine speed, even though the load may vary

(B). Maintain, within certain limits, a desired oil level, even though the load may vary

(C). Maintain, within certain limits, a desired engine temperature, even though the load may vary

(D). Regulate the speed when there is no load

175. With the air vane governor, air pressure against the air vane tends to ____________ the throttle.

(A). Open

(B). Close

(C). Both A and B

(D). None of the above

176. In a mechanical governor, it is the force of the ____________ that tends to open and close the throttle.

(A). Counterweights

(B). Springs

(C). Centrifugal force

(D). Valve lobes

177. A loss of compression can be a result of:

(A). Cylinder head gasket blown or leaking

(B). Valves sticking or not seating properly

(C). Piston rings not sealing

(D). All of the above

178. Do not use a sealer of any kind on the head gasket.

(A). True

(B). False

179. Head bolts should not be tightened in sequence.

(A). True

(B). False

180. An engine that will run a few minutes, stall, restart, run a few more minutes and stall may have a

(A). Restricted main jet

(B). Dirty fuel filter

(C). Plugged fuel tank vent

(D). Dirty air filter

181. Exhaust valve faces should be resurfaced to:

(A). 35 degrees

(B). 45 degrees

(C). 25 degrees

(D). 10 degrees

182. Some engines use the same size spring for intake and exhaust valves, while others:

(A). Use a larger spring on the intake

(B). Use a larger spring on the intake and exhaust

(C). Use a smaller spring on the exhaust

(D). Use a larger spring on the exhaust

183. The three types of valve spring retainers are:

(A). Pin, collar, retainer

(B). Collar, key, ball

(C). Ball, retainer, pin

(D). Key, ball, socket

184. The life of a standard exhaust valve is:

(A). Often lengthened by heating

(B). Often shortened by cooling

(C). Often shortened by burning

(D). Often lengthened by carbon deposits

185. Exhaust valve life can be extended by:

(A). A rotocoil rotator

(B). A cobalt exhaust valve

(C). Grinding the valve every 25 hours

(D). Both A and B

186. The starter spring should be wound counter clockwise.

(A). True

(B). False

187. Do not run the engine without the screen or screws assembled to the clutch.

(A). True

(B). False

188. The four purposes of oil are:

(A). Cleans, cools, moisturizes, gums

(B). Cleans, lubricates, heats, cools

(C). Cleans, cools, lubricates, seals

(D). Lubricates, cools, seals, coats

189. Most Briggs & Stratton engines are lubricated with a:

(A). Gear driven splash oil slinger

(B). Connecting rod dipper

(C). All of the above

(D). None of the above

190. Detergent oils should be used because it can:

(A). Retard the formation of gum and varnish

(B). Clean the burned oil

(C). Retard carbon deposits

(D). Thin the oil for greater coverage

191. The breathers’ function is to:

(A). Find fresh air

(B). Maintain crankcase vacuum

(C). Maintain oil pressure

(D). Maintain the air fuel mixture

192. A partial vacuum must be maintained in the crankcase:

(A). To prevent oil from being forced into the engine

(B). To prevent air from getting into the crankcase

(C). To increase fuel efficiency

(D). To prevent oil from being forced out of the engine

193. The oil slinger is driven by the:

(A). Cam gear

(B). Crank gear

(C). Piston

(D). Connecting rod

194. The purpose of the oil guard is to:

(A). Protect the oil from getting old

(B). Clean the oil

(C). Prevent engine damage because of a low oil shut off system

(D). Prevent engine damage because of a high oil shut off system

195. An oil dipper is connected to the:

(A). Connecting rod

(B). Cam shaft

(C). Crankshaft

(D). Valves

196. To adjust the governor on a model 126332 OHV engine, you should:

(A). Put on full throttle, turn adjustment arm counterclockwise, then retighten the bolt.

(B). Put on full throttle, turn adjustment arm clockwise, then retighten the bolt.

(C). Put on half throttle, turn adjustment arm clockwise, then retighten the bolt.

(D). Put on half throttle, turn adjustment arm counterclockwise, then retighten the bolt.

197. If the crankpin bearing in the connecting rod is worn:

(A). The rod should be replaced

(B). The rod should be ground

(C). The bearing should be replaced

(D). All of the above

198. Do not attempt to "file" or "fit" the rod.

(A). True

(B). False

199. On pistons with a notch on the head:

(A). The notch should face the back of the engine

(B). The notch should face the magneto

(C). The notch should face the carburetor

(D). The notch can go in any direction

200. The rings should be installed in what order?

(A). Compression ring, oil ring, smoke ring

(B). Oil ring, center compression ring, bottom compression ring

(C). Center compression ring, oil ring, top compression ring

(D). Oil ring, center compression ring, top compression ring

201. Rod caps can be installed in any direction.

(A). True

(B). False

202. Always resize exactly __________ over the standard bore size.

(A). .010"

(B). .020"

(C). .030"

(D). Any of the above

203. If the engine is bored with a boring bar, a hone must be used after the boring operation to:

(A). Get the last one hundredth of an inch

(B). Produce the proper cylinder wall finish

(C). Produce an oblong shape

(D). Remove the boring oils

204. Synchro balancing of an engine is accomplished done by:

(A). Using counterweights that rotate in a direction opposite from the crankshaft counterweights

(B). Using a counterweight that oscillates opposite to the direction of the piston

(C). Either A or B

(D). Neither A nor B

205. The operating temperature of an air cooled engine may vary with:

(A). Changes in air temperature

(B). The load

(C). The speed

(D). All of the above

206. During the intake stroke the burnt air fuel mixture is forced out of the engine.

(A). True

(B). False

207. During the compression stroke, the air fuel mixture becomes greatly compressed in the small space between the top of the piston and the cylinder head.

(A). True

(B). False

208. During the power stroke the force of the expanding gas forces the piston up.

(A). True

(B). False

209. During the exhaust stroke, both valves are open.

(A). True

(B). False

210. Compression ratios tell us the horsepower of an engine.

(A). True

(B). False

211. The most important factor in good compression is:

(A). Head gasket

(B). Valves

(C). Rings

(D). All of the above

212. When the engine is operating at 3000 RPM, each valve opens and closes in about:

(A). 1/50 of a second

(B). 1 second

(C). 1/2 of a second

(D). 1/25 of a second

213. The heat on the head of the exhaust valve is:

(A). 1,200 degrees

(B). 3,000 degrees

(C). 2,500 degrees

(D). 1,000 degrees

214. Valves have to seal well enough to withstand pressures up to __________ pounds per square inch.

(A). 1000

(B). 100

(C). 10

(D). 500

215. The intake valve is cooled by:

(A). The flywheel

(B). The incoming air fuel mixture

(C). Oil

(D). Water

216. The cylinder head, the cylinder, and the top of the piston are cooled by:

(A). Water from the radiator

(B). Air from the flywheel fan

(C). Oil from the crankcase

(D). Both B and C

217. In a one cylinder engine, one bad valve will not cause a drop in horsepower or cause the engine to stop entirely.

(A). True

(B). False

218. An accumulation of carbon or fuel lead either on the valve stem or on the valve face, or insufficient tappet clearance, can cause:

(A). Valve seat burning

(B). Valve stem bending

(C). Valve tappet burning

(D). Weak spark

219. A valve with a sunken head caused by operating at too high a temperature with too strong a spring is known as a:

(A). Necked valve

(B). Stuck valve

(C). Dished valve

(D). Cracked valve

220. A valve that has the stem directly beneath the head eaten away badly by heat is called a:

(A). Necked valve

(B). Stuck valve

(C). Dished valve

(D). Cracked valve

221. Valve sticking is caused by:

(A). Old oil

(B). Fuel lead, gum, or varnish

(C). Water in the fuel or oil

(D). None of the above

222. If burning occurs in a rather limited area on the valve face, it indicates:

(A). Sticking because of fuel lead

(B). Crack on the valve head

(C). Necked valve

(D). Something caused the valve to tip

223. The important parts of the valve are:

(A). Head, margin, face, seat

(B). Head, margin, stem, tappet

(C). Head, margin, face, stem

(D). Stem, face, guide, head

224. The margin is the edge of the valve head.

(A). True

(B). False

225. A valve with too thin a margin will:

(A). Not be able to withstand gum and varnish and will stick

(B). Not be able to withstand the heat and will quickly crack and burn

(C). Be able to open and close quickly

(D). Not be able to open at all

226. Use a fine grinding compound to lubricate the valves.

(A). True

(B). False

227. After grinding the valve, the tappet clearance will usually be too small.  To correct this, you should:

(A). Replace the valve

(B). Grind the lobes on the crankshaft

(C). Grind the end of the valve stem

(D). Grind the tappet

228. The basic purpose of the carburetor is to produce a mixture of fuel and air on which the engine will operate.

(A). True

(B). False

229. Atmospheric pressure:

(A). Is a potent force which tends to increase itself in any given area

(B). Is a constant force which tends to disrupt itself in any given area

(C). Applies pressure only during the intake stroke

(D). Is a constant potent force which tends to equalize itself in any given area

230. Atmospheric pressure is commonly between:

(A). 13 to 15 pounds per square inch

(B). 5 to 9 pounds per square inch

(C). 25 to 30 pounds per square foot

(D). 1 to 2 pounds per square inch

231. The __________ shape is carefully designed to produce certain air flow patterns.

(A). Airfoil

(B). Venturi

(C). Throttle

(D). Choke

232. In a gravity feed system, the tank is above the carburetor and fuel flows by gravity.

(A). True

(B). False

233. In a flow jet carburetor there is an:

(A). Air vent hole in the tank cap to let air flow in as fuel flows out and a hole in the bowl so that air flows out as fuel flows in

(B). Air vent hole in the tank cap to let air flow out as fuel flows out and a hole in the bowl so that air flows out as fuel flows in

(C). Air vent hole in the tank cap to let air flow in as fuel flows out and a hole in the bowl so that air flows out as fuel flows out

(D). Air vent hole in the tank cap to let air flow out as fuel flows out and a hole in the bowl so that air flows in as fuel flows out

234. The ideal combustion mixture in the flo-jet carburetor is about:

(A). 14 to 15 lbs of air, in weight, to twenty pounds of gasoline

(B). 1 to 2 lbs of air, in weight, to twenty pounds of gasoline

(C). 14 to 15 lbs of air, in weight, to one pound of gasoline

(D). 70 to 80 lbs of air, in weight, to one pound of gasoline

235. An air-gasoline combustion mixture that is too lean causes:

(A). Overheating, detonation, and short valve clearance

(B). Overheating, detonation, and short valve life

(C). Overheating, detonation, and long valve life

(D). Cooling, cleaning, and long valve life

236. A flat disc called a butterfly that is located in the throat of the carburetor above the venturi and that allows for different speeds is the:

(A). Throttle

(B). Choke

(C). Needle valve

(D). Air vane

237. With the choke you can close, or almost close, the air horn and get:

(A). A high pressure in the crankcase; thus a rush of fuel is obtained with a relatively small amount of air

(B). A low pressure area in the venturi and throat; thus a rush of fuel is obtained with a relatively small amount of air

(C). A low pressure area in the crankcase; thus a rush of fuel is obtained with a relatively small amount of air

(D). None of the above

238. With vacu-jet carburetors, the fuel tank is below the carburetor.  Therefore, the force of atmospheric pressure must be employed.

(A). True

(B). False

239. The vacu-jet carburetor should be adjusted with the fuel tank:

(A). Full

(B). Three-quarters full

(C). One-third full

(D). Half full

240. The amount of fuel at operating speed is metered by the:

(A). Needle valve and throttle

(B). Throttle and choke

(C). Needle valve and seat

(D). Seat and intake valve

241. In the pulsa jet carburetor, the vacuum created by the diaphragm pump:

(A). Pulls gas up the suction pipe into the pocket and then to the fuel cap

(B). Pulls gas up the suction pipe into the fuel cap and then to the pocket

(C). Pulls gas up the suction pipe into the diaphragm and then to the venturi

(D). Pulls gas up the suction pipe into the venturi

242. The use of lead free gasoline results in:

(A). Fewer combustion deposits and shorter valve life

(B). More combustion deposits and shorter valve life

(C). More combustion deposits and longer valve life

(D). Fewer combustion deposits and longer valve life

243. If you must use gasoline with alcohol, the alcohol content should be:

(A). Less than 1 %

(B). Less than 10 %

(C). Less than 25 %

(D). More than 15 %

244. A 3HP engine operating at 2600 RPM uses:

(A). About 410 cubic feet of air an hour entering at a rate of about 24 miles an hour

(B). About 140 cubic feet of air an hour entering at a rate of about 20 miles an hour

(C). About 390 cubic feet of air an hour entering at a rate of about 24 miles an hour

(D). About 390 cubic feet of air an hour entering at a rate of about 20 miles an hour

245. A magneto consists of the following two circuits:

(A). First and second

(B). First and secondary

(C). Primary and secondary

(D). Primary and second

246. Both circuits in a magneto have windings which:

(A). Surround the same iron core and the magnets on the flywheel and rotor act on both circuits

(B). Surround the same iron core and the magnets on the flywheel or rotor act on both circuits

(C). Surround the same plastic core and the magnets on the flywheel or rotor act only on the primary circuit

(D). Surround the same iron core and the magnets on the flywheel or rotor act only on the primary circuit

247. Current can be induced in each circuit in the magneto by:

(A). Changing the speed of the engine

(B). Changing the magnetism

(C). Changing the spark plug

(D). Changing the points

248. The rapid changes in magnetism produces:

(A). 150 volts in the secondary circuit

(B). 170 volts in the primary circuit

(C). 170 volts in the secondary circuit

(D). 150 volts in the primary circuit

249. Spark plug cables are molded into the coil so that:

(A). Dirt does not get in and wear out the windings

(B). The cable is protected from being pulled out

(C). Moisture cannot short out the spark

(D). The spark does not jump out the side of the coil

250. The flywheel key is:

(A). A locator and a driver

(B). A driver and not a locator

(C). Not a locator or a driver

(D). A locator and not a driver

251. The purpose of the governor is to:

(A). Prevent overspeeding

(B). Maintain a desired speed regardless of load

(C). Prevent underspeeding

(D). Maintain a desired oil level

252. The pneumatic governor is operated by:

(A). The force of air from the flywheel

(B). The throttle

(C). Cam gear

(D). Crankshaft gear

253. Ignition timing on aluminum engines is controlled by:

(A). The location of the cam shaft to the crankshaft

(B). The location of the armature

(C). The location of the flywheel and crankshaft keyways

(D). Timing is not important

254. What is the most likely problem if an engine will not start, kicks back when you pull the rope, and the valve clearance is correct?

(A). The engine is not getting enough fuel

(B). The kill switch is in the off position

(C). The flywheel key could be sheared

(D). The kill wire is grounded

255. The capacity of the oil foam air cleaner is:

(A). 1/2 quart

(B). Adequate for a full season's use without cleaning with average use

(C). Adequate for 5 hours operation without cleaning with average use

(D). One quart

256. To clean a cartridge type air cleaner:

(A). Tap the cartridge (top and bottom) on a flat surface

(B). Wash in non sudsing detergent and flush from the inside until the water is clear

(C). Replace, do not clean

(D). Both A and B

257. The three types of remote controls are:

(A). Governor control, Throttle control, Choke-A-Matic control

(B). Governor control, Spark control, Choke-A-Matic

(C). Spark control, Throttle control, Choke-A-Matic

(D). Spark control, Throttle control, Governor control

258. The three types of air cleaners on small engines are:

(A). Oil bath, Dual element, Rubber type

(B). Dual element, Cartridge type, Oil bath

(C). Foam, Cartridge type, Oil dip

(D). Oil dip, Dual element, Cartridge type

259. Valve condition is more important in a one cylinder engine than it is in a multi-cylinder engine.

(A). True

(B). False

260. Fair compression is necessary in order to obtain the full horsepower of an engine.

(A). True

(B). False

261. An engine that has good compression, carburetion, and ignition should operate efficiently.

(A). True

(B). False

262. The intake valve clearance is:

(A). .005 to .010

(B). .002 to .003

(C). .005 to .007

(D). .007 to .010

263. The exhaust valve clearance is:

(A). .007 to .009

(B). .005 to .007

(C). .010 to .012

(D). None of the above

264. What is pre-ignition?

(A). When your engine is equipped with a secondary ignition system

(B). Abnormal combustion before the timed spark

(C). Pre-ignition of the fuel which allow the engine to warm up quicker

(D). When the points are set wrong and they open prematurely

265. TDC is an abbreviation for:

(A). Too Dry of a Crankcase

(B). Top Dead Center

(C). Top Displacement of Cylinder

(D). None of the above

266. Excessive spark plug electrode clearance will cause:

(A). A high speed miss

(B). A low speed miss

(C). Overspeeding

(D). A hot engine

267. The RJ19LM spark plug from the L-Head engines is interchangeable with the RBL-15Y off the OHV engines.

(A). True

(B). False

268. Any number of engines from the same manufacturer could all have the same model number.

(A). True

(B). False

269. Fuel pouring out of a flo jet carburetor can be a sign of which of the following problems?

(A). Dirty jet

(B). Fuel needle stuck open

(C). Cracked float

(D). Both B & C

270. The positive terminal of a battery is connected to:

(A). The frame

(B). The alternator

(C). The generator

(D). The switch

271. For every thousand feet above sea level, power is reduced by approximately what percent

(A). 1.6%

(B). 3.5%

(C). 10%

(D). 15%

272. As a general rule, engine horsepower will

(A). Increase as elevation increases

(B). Increase as temperature increases

(C). Decrease as elevation increases

(D). Decrease as barometric pressure increases

273. When does the exhaust valve closes and the intake valve start to open?

(A). At the end of the compression stroke

(B). At the end of the intake stroke

(C). At the end of the power stroke

(D). At the end of the exhaust stroke

274. Loose carburetor mounting bolts on a small engine will cause:

(A). The engine to flood

(B). The engine to starve for fuel

(C). The engine to be easier to start

(D). Either A or B

275. What is the most likely solution when a small engine blows out a lot of black smoke when running and runs poorly?

(A). Oil change

(B). Air filter change

(C). Valve adjustment

(D). None of the above.

276. When you loosen the fuel bowl screw to check for fuel getting to the carburetor., the fuel flows out and beads up on the floor. What does this indicate?

(A). You have a good fuel supply and need to check for something else.

(B). Water is in the fuel system and the system should be drained and cleaned

(C). The fuel is too thick and needs to be thinned.

(D). The fuel is too cold and should be warmed up.

277. What is the most likely problem if the engine will run but has a lot of blue smoke coming out of the muffler.

(A). The valves are worn and should be replaced.

(B). The piston rings are worn and should be replaced.

(C). The sparkplug is faulty and should be replaced.

(D). The connecting rod is worn and should be replaced.

278. Which of the following could lead to weak engine compression?

(A). Piston rings, valves, head gasket

(B). Piston rings, fuel supply, head gasket

(C). Piston rings, ignition system, valves

(D). Valves, kill switch, fuel valve

279. Valves that are adjusted so that there is little or no valve clearance will:

(A). Increase engine compression, thus produce more power

(B). Decrease engine compression, thus increasing engine power

(C). Decrease engine compression and may cause the engine not to start

(D).

280. While troubleshooting the ignition system on a small engine, you find that the system will not fire the sparkplug unless you disconnect the kill wire from the ignition coil. The most likely problem is:

(A). You have a defective coil and should replace it

(B). You need to re-set the air gap between the coil and flywheel

(C). The coil is good, the problem is with the engine kill system

(D). None of the above

281. To check for compression on small engines:

(A). Spin the flywheel counter clockwise and it should bounce back indicating good compression

(B). Spin the flywheel clockwise and it should spin with no resistance

(C). Spin the flywheel counter clockwise and it should spin with no resistance

(D). Remove the sparkplug first and spin the flywheel clockwise; it should bounce back indicating good compression

282. What is the most likely problem if the engine is flooding and dripping fuel from the carburetor?

(A). The float could be filled with fuel

(B). The needle valve could be sealed completely

(C). The fuel tank could be over filled

(D). The engine oil level could be incorrect

283. You are starting an engine that has just been over hauled. What is the most likely problem if a small engine starts but runs too fast?

(A). The throttle control lever is set too low

(B). The governor is not set properly

(C). The fuel valve is not set properly

(D). The valves are not set properly

284. What is the most likely problem if you check the oil in a small engine and find that it is over full and smells like gasoline?

(A). The ignition system is probably at fault and needs repair

(B). The fuel system is probably at fault and needs repair

(C). The engine has compression problems

(D). Nothing is wrong. This is normal.

285. Flo jet carburetors must be mounted __________ the fuel tank to operate.

(A). Above

(B). Below

(C). Neither A nor B.

286. Valve stem clearances and / or valve lash should be checked and adjusted with the valve stem caps off the engine.

(A). True

(B). False

287. If the flywheel is not torqued to specifications, it may result in a sheared flywheel key.

(A). True

(B). False

288. The term used to describe the period of time both valves are open during engine operation is:

(A). Dual timing

(B). Valve overlap

(C). Valve timing

(D). 10% valve separation.

289. The governed speed of Briggs & Stratton engines can be altered by changing the colored governor spring.

(A). True

(B). False

290. Mounting a(n) __________ would allow you to attach the fuel tank anywhere near the engine.

(A). Auxiliary tank

(B). Fuel pump

(C). Extra distributor

(D). None of these.

Louisiana Small Engines CDE TestBank

Test Answer Key

C:\data\CDE-Tests\Small.tst

Test ID: 74

1.  Correct Answer(s): D

2.  Correct Answer(s): A

3.  Correct Answer(s): A

4.  Correct Answer(s): B

5.  Correct Answer(s): B

6.  Correct Answer(s): A

7.  Correct Answer(s): D

8.  Correct Answer(s): D

9.  Correct Answer(s): B

10.  Correct Answer(s): B

11.  Correct Answer(s): B

12.  Correct Answer(s): C

13.  Correct Answer(s): A

14.  Correct Answer(s): D

15.  Correct Answer(s): C

16.  Correct Answer(s): A

17.  Correct Answer(s): C

18.  Correct Answer(s): C

19.  Correct Answer(s): D

20.  Correct Answer(s): B

21.  Correct Answer(s): B

22.  Correct Answer(s): A

23.  Correct Answer(s): A

24.  Correct Answer(s): C

25.  Correct Answer(s): C

26.  Correct Answer(s): B

27.  Correct Answer(s): C

28.  Correct Answer(s): D

29.  Correct Answer(s): C

30.  Correct Answer(s): A

31.  Correct Answer(s): B

32.  Correct Answer(s): A

33.  Correct Answer(s): A

34.  Correct Answer(s): B

35.  Correct Answer(s): D

36.  Correct Answer(s): D

37.  Correct Answer(s): B

38.  Correct Answer(s): C

39.  Correct Answer(s): A

40.  Correct Answer(s): A

41.  Correct Answer(s): B

42.  Correct Answer(s): D

43.  Correct Answer(s): C

44.  Correct Answer(s): D

45.  Correct Answer(s): D

46.  Correct Answer(s): C

47.  Correct Answer(s): B

48.  Correct Answer(s): D

49.  Correct Answer(s): C

50.  Correct Answer(s): A

51.  Correct Answer(s): C

52.  Correct Answer(s): D

53.  Correct Answer(s): C

54.  Correct Answer(s): C

55.  Correct Answer(s): C

56.  Correct Answer(s): B

57.  Correct Answer(s): B

58.  Correct Answer(s): C

59.  Correct Answer(s): D

60.  Correct Answer(s): B

61.  Correct Answer(s): A

62.  Correct Answer(s): B

63.  Correct Answer(s): A

64.  Correct Answer(s): D

65.  Correct Answer(s): B

66.  Correct Answer(s): B

67.  Correct Answer(s): B

68.  Correct Answer(s): C

69.  Correct Answer(s): C

70.  Correct Answer(s): A

71.  Correct Answer(s): D

72.  Correct Answer(s): A

73.  Correct Answer(s): A

74.  Correct Answer(s): C

75.  Correct Answer(s): B

76.  Correct Answer(s): A

77.  Correct Answer(s): A

78.  Correct Answer(s): A

79.  Correct Answer(s): C

80.  Correct Answer(s): C

81.  Correct Answer(s): A

82.  Correct Answer(s): A

83.  Correct Answer(s): D

84.  Correct Answer(s): A

85.  Correct Answer(s): A

86.  Correct Answer(s): A

87.  Correct Answer(s): B

88.  Correct Answer(s): A

89.  Correct Answer(s): A

90.  Correct Answer(s): B

91.  Correct Answer(s): C

92.  Correct Answer(s): D

93.  Correct Answer(s): B

94.  Correct Answer(s): B

95.  Correct Answer(s): B

96.  Correct Answer(s): C

97.  Correct Answer(s): D

98.  Correct Answer(s): C

99.  Correct Answer(s): C

100.  Correct Answer(s): B

101.  Correct Answer(s): D

102.  Correct Answer(s): C

103.  Correct Answer(s): B

104.  Correct Answer(s): C

105.  Correct Answer(s): B

106.  Correct Answer(s): A

107.  Correct Answer(s): C

108.  Correct Answer(s): C

109.  Correct Answer(s): D

110.  Correct Answer(s): B

111.  Correct Answer(s): B

112.  Correct Answer(s): A

113.  Correct Answer(s): B

114.  Correct Answer(s): D

115.  Correct Answer(s): B

116.  Correct Answer(s): C

117.  Correct Answer(s): B

118.  Correct Answer(s): A

119.  Correct Answer(s): B

120.  Correct Answer(s): D

121.  Correct Answer(s): A

122.  Correct Answer(s): D

123.  Correct Answer(s): C

124.  Correct Answer(s): D

125.  Correct Answer(s): B

126.  Correct Answer(s): B

127.  Correct Answer(s): A

128.  Correct Answer(s): A

129.  Correct Answer(s): C

130.  Correct Answer(s): B

131.  Correct Answer(s): A

132.  Correct Answer(s): D

133.  Correct Answer(s): B

134.  Correct Answer(s): C

135.  Correct Answer(s): B

136.  Correct Answer(s): C

137.  Correct Answer(s): C

138.  Correct Answer(s): A

139.  Correct Answer(s): C

140.  Correct Answer(s): A

141.  Correct Answer(s): B

142.  Correct Answer(s): A

143.  Correct Answer(s): A

144.  Correct Answer(s): C

145.  Correct Answer(s): A

146.  Correct Answer(s): A

147.  Correct Answer(s): A

148.  Correct Answer(s): D

149.  Correct Answer(s): B

150.  Correct Answer(s): A

151.  Correct Answer(s): C

152.  Correct Answer(s): D

153.  Correct Answer(s): D

154.  Correct Answer(s): C

155.  Correct Answer(s): A

156.  Correct Answer(s): A

157.  Correct Answer(s): B

158.  Correct Answer(s): C

159.  Correct Answer(s): C

160.  Correct Answer(s): B

161.  Correct Answer(s): A

162.  Correct Answer(s): A

163.  Correct Answer(s): C

164.  Correct Answer(s): B

165.  Correct Answer(s): B

166.  Correct Answer(s): A

167.  Correct Answer(s): C

168.  Correct Answer(s): A

169.  Correct Answer(s): A

170.  Correct Answer(s): B

171.  Correct Answer(s): A

172.  Correct Answer(s): A

173.  Correct Answer(s): D

174.  Correct Answer(s): A

175.  Correct Answer(s): B

176.  Correct Answer(s): A

177.  Correct Answer(s): D

178.  Correct Answer(s): A

179.  Correct Answer(s): B

180.  Correct Answer(s): C

181.  Correct Answer(s): B

182.  Correct Answer(s): D

183.  Correct Answer(s): A

184.  Correct Answer(s): C

185.  Correct Answer(s): D

186.  Correct Answer(s): A

187.  Correct Answer(s): A

188.  Correct Answer(s): C

189.  Correct Answer(s): C

190.  Correct Answer(s): A

191.  Correct Answer(s): B

192.  Correct Answer(s): D

193.  Correct Answer(s): A

194.  Correct Answer(s): C

195.  Correct Answer(s): A

196.  Correct Answer(s): A

197.  Correct Answer(s): A

198.  Correct Answer(s): A

199.  Correct Answer(s): B

200.  Correct Answer(s): D

201.  Correct Answer(s): B

202.  Correct Answer(s): D

203.  Correct Answer(s): B

204.  Correct Answer(s): C

205.  Correct Answer(s): D

206.  Correct Answer(s): B

207.  Correct Answer(s): A

208.  Correct Answer(s): B

209.  Correct Answer(s): B

210.  Correct Answer(s): B

211.  Correct Answer(s): D

212.  Correct Answer(s): A

213.  Correct Answer(s): C

214.  Correct Answer(s): D

215.  Correct Answer(s): B

216.  Correct Answer(s): D

217.  Correct Answer(s): B

218.  Correct Answer(s): A

219.  Correct Answer(s): C

220.  Correct Answer(s): A

221.  Correct Answer(s): B

222.  Correct Answer(s): D

223.  Correct Answer(s): C

224.  Correct Answer(s): A

225.  Correct Answer(s): B

226.  Correct Answer(s): B

227.  Correct Answer(s): C

228.  Correct Answer(s): A

229.  Correct Answer(s): D

230.  Correct Answer(s): A

231.  Correct Answer(s): B

232.  Correct Answer(s): A

233.  Correct Answer(s): A

234.  Correct Answer(s): C

235.  Correct Answer(s): B

236.  Correct Answer(s): A

237.  Correct Answer(s): B

238.  Correct Answer(s): A

239.  Correct Answer(s): D

240.  Correct Answer(s): C

241.  Correct Answer(s): A

242.  Correct Answer(s): D

243.  Correct Answer(s): B

244.  Correct Answer(s): C

245.  Correct Answer(s): C

246.  Correct Answer(s): A

247.  Correct Answer(s): B

248.  Correct Answer(s): B

249.  Correct Answer(s): C

250.  Correct Answer(s): D

251.  Correct Answer(s): B

252.  Correct Answer(s): A

253.  Correct Answer(s): C

254.  Correct Answer(s): C

255.  Correct Answer(s): B

256.  Correct Answer(s): D

257.  Correct Answer(s): A

258.  Correct Answer(s): B

259.  Correct Answer(s): A

260.  Correct Answer(s): B

261.  Correct Answer(s): A

262.  Correct Answer(s): C

263.  Correct Answer(s): A

264.  Correct Answer(s): B

265.  Correct Answer(s): B

266.  Correct Answer(s): A

267.  Correct Answer(s): B

268.  Correct Answer(s): A

269.  Correct Answer(s): D

270.  Correct Answer(s): B

271.  Correct Answer(s): B

272.  Correct Answer(s): C

273.  Correct Answer(s): D

274.  Correct Answer(s): B

275.  Correct Answer(s): B

276.  Correct Answer(s): B

277.  Correct Answer(s): B

278.  Correct Answer(s): A

279.  Correct Answer(s): C

280.  Correct Answer(s): C

281.  Correct Answer(s): A

282.  Correct Answer(s): A

283.  Correct Answer(s): B

284.  Correct Answer(s): B

285.  Correct Answer(s): B

286.  Correct Answer(s): B

287.  Correct Answer(s): A

288.  Correct Answer(s): B

289.  Correct Answer(s): A

290.  Correct Answer(s): B