PRACTICUM 2

E

A

IDENTIFY THE CLUTCH DRUM BEARING

D

C

B

C

D

Identify the carburetor

A

B

E

- What is the correct cutting sequence for this situation (compression is on top). You will cut close to the stump so the log is not elevated over your head.

- Cut 1/3 from the top until it feels like the tree may pinch the bar then finish from the bottom. Stagger the finishing cut by 1/2”- 1” to the right of the first cut.

- Cut 1/3 from the top until it feels like the tree may pinch the bar, then finish from the bottom. Make the cuts meet.

- Cut 1/3 from the bottom and finish from the top staggering the cuts by 1”

- Cut 1/3 from the bottom until it feels like the tree may pinch the bar and finish from the top. Make the cuts meet.

- Cut all the way through from bottom to the top.

- What does this part do?

- Catches the chain if it breaks or derails

- Sharpens the chain as it goes by

- Lubricates the guide bar with chain oil

- Stops the chain when the brake is engaged

- None of the above

- What do these fins do?

- Lubricate the piston

- Keep the engine from being too loud

- Keep emissions safe

- Cool the engine

- None of the above

- This tree is 20” in diameter. According to the Husqvarna manual recommendations, how deep should this notch cut go back into the tree?

- 3”-4”

- 7”- 8”

- 9” - 10”

- 13” - 14”

- 15” - 16”

- Why would you need to cut a tree down like this?

- The saw is not powerful

- The tree diameter is a little larger than the guide bar length

- The tree diameter is twice the size of the guide bar length

- Tree diameter is smaller than the guide bar length

- What width should the hinge wood be on this tree?

A 1”

B 2”

C 3”

D 4”

E 24”

DBH 30”

- When limbing thick heavy branches, what is the basic rule?

- Cut the branch flush with the trunk

- Cut the branch off the trunk leaving 3”

- Stand up on the tree and limb

- Work from the outside of the bigger limb and in towards the trunk

- Work from the inside of the bigger limb and out towards the end

- Why would you do this?

- You couldn’t tell where the tension/compression was

- You just sharpened your saw

- Your tired and don’t want to work anymore

- Limb tension needed to be released

- None of the above

- Identify the chisel of the saw tooth

D

C

B

A

- Which is the safest and most powerful way to fell a tree?

- Using felling wedges

- Using foot breaking bars

- Using Impact bars

- Using breaking bars

- Using a rope and a winch

- What tree felling method is this?

- Cutting with bore

- Turn around method

- Safe corner method

- Saved edge method

- Felling cut straight from behind

- What is your path of retreat?

- Straight line directly behind the felling cut

- 45 degree angled line in front of the directional notch

- 45 degree angled line on either side behind the felling cut

- Straight 90 degree line right or left of the felling cut

- Straight line in front of the directional notch

- Locks the throttle in high speed

- Kills the engines

- Prevents the engine from flooding

- Prevents accidental throttle revving

- None of the above

A

What does “A” do?

- Which area would pinch your saw if you tried to cut it all the way through?

A

B

- What could happen if you completely filed down the rakers?

- The teeth would cut too deeply into the wood

- The chances of kickback would greatly increase

- The load on the engine of the saw could result in high vibrations

- Chainsaw is exposed to unnecessary stress

- All of the above

- If sharpening your saw in your shop, what is the first thing you should do?

A. Lock the guide bar in a bench vice

B. File down the depth gauges

C. Begin filing at 25-25 degrees

D. Deactivate the chain brake

E. None of the above

- If you do NOT have the skills, capability or experience to cut this leaning tree, what is the smartest/safest thing you should do in this scenario?

A Rope off the area with safety flagging and leave

B Go borrow a tractor and try to cut it

C Cut the trees on the ground and leave the leaner alone

D Just go slow and hope everything turns out OK

A California

B Key notch

C Humboldt

D Open face

An inverse directional notch like this is called a what?

1C

2D

3A

4A

5D

6A

7C

8C

9D

10D

11D

12E

13B

14C

15D

16B

17E

18A

19A

20C